SOFTWARE REQUIREMENTS SPECIFICATION

Clinker Bucket Elevator Vision Monitoring System

MVP: local physical server deployment

Document Status: Draft v0.5 — revised to match the approved MVP baseline

| Version | Status | Summary |
| --- | --- | --- |
| 0.5 | Draft | Rebased on the physical HP server, direct NVR Ethernet connection, Docker deployment, CAM-1 godet MVP, local web viewer, and local evidence storage. |

# 1 Introduction

## 1.1 Purpose

This document specifies the MVP requirements for monitoring a bucket elevator at a cement plant. The system receives camera video through a Dahua network video recorder, samples frames, sends full frames to an external model service, stores qualifying alerts and evidence, and presents the results in a local web viewer.

## 1.2 MVP scope

Six cameras are connected to the NVR, but only camera/channel CAM-1 is enabled in the MVP. CAM-1 observes godets. The initial model fault scope is misalignment and crack detection; the model returns the fault type. The architecture keeps the other five cameras configurable for later phases.

## 1.3 Explicit MVP boundaries

The system runs on one physical HP server with Ubuntu Server LTS; no VMware VM is required.

The NVR connects to the server over Ethernet on a private camera network; it is not assumed to be on the plant OT network.

The pipeline and model run as Docker containers on the same server.

The pipeline sends full-frame JPEG images and performs no crop. The model owns crop, resize, normalization, and inference.

The web viewer is opened directly on the server. There is no mobile app, push notification, external backend, user authentication, or TLS in the MVP.

Evidence and SQLite data are stored on the server's local 500 GB disk. NAS/NFS is deferred.

Frames that the model does not process are not replayed. The NVR recording remains the source of camera history.

# 2 System context

The six plant cameras feed the Dahua NVR. The NVR records the camera streams and exposes channel streams. The Ubuntu server pulls CAM-1 over Ethernet. The pipeline samples approximately one frame every two seconds, calls the model service, and displays/stores only qualifying detections.

| Component | MVP responsibility |
| --- | --- |
| Camera/NVR | Capture and retain camera video; expose CAM-1 as RTSP. |
| Pipeline container | RTSP ingestion, sampling, full-frame JPEG preparation, queueing, model client, alert persistence, evidence overlay, local REST API, and web viewer. |
| Model container | Python/OpenCV preprocessing and godet inference; returns raw detections and model version over gRPC. |
| Server disk | SQLite database, local evidence JPEGs, Docker images, logs, and configuration. |
| Operator | Uses a browser directly on the server to review the local alert feed and evidence. |

# 3 Functional requirements

## 3.1 NVR and video ingestion

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-ING-1 | The system shall connect to the NVR's CAM-1 RTSP stream using the plant-provided NVR IP, credentials, and channel URL. | Must |
| FR-ING-2 | The system shall run on the physical Ubuntu server and shall not require a VM. | Must |
| FR-ING-3 | The system shall decode CAM-1 frames and send full frames to the queue without ROI cropping. | Must |
| FR-ING-4 | The system shall detect and log NVR, RTSP, authentication, and decode failures. | Must |
| FR-ING-5 | The configuration shall contain all six camera entries, with CAM-1 enabled and CAM-2 through CAM-6 disabled in the MVP. | Must |

## 3.2 Sampling and queueing

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-QUE-1 | The pipeline shall sample approximately one frame per two seconds from CAM-1; the interval shall be configurable. | Must |
| FR-QUE-2 | Each enabled camera shall have a bounded in-memory queue. | Must |
| FR-QUE-3 | When the queue is full, the oldest frame shall be discarded so ingestion does not build an unlimited backlog. | Must |
| FR-QUE-4 | Frame drops shall be counted and visible in logs or metrics. | Should |

## 3.3 Model interface

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-INF-1 | The pipeline shall call the model through the agreed gRPC streaming contract. | Must |
| FR-INF-2 | The pipeline shall send full-frame JPEG bytes, camera_id, frame_id, captured_at, and sequence_no. | Must |
| FR-INF-3 | The model service shall perform dynamic crop, resize, normalization, and inference. | Must |
| FR-INF-4 | For the MVP, detections shall target GODET and shall include CRACK or MISALIGNMENT when applicable, raw confidence, full-frame normalized bounding box, and model_version. | Must |
| FR-INF-5 | The pipeline shall start with a confidence threshold of 0.85 to reduce false positives; the value shall be configurable. | Must |
| FR-INF-6 | A model timeout, unavailable model, or invalid response shall be logged as an error and shall not be treated as a normal empty detection. | Must |

## 3.4 Alerts and local web viewer

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-ALT-1 | Each qualifying model detection shall create one alert. No deduplication or cooldown is required in the MVP. | Must |
| FR-ALT-2 | Alerts shall be stored in SQLite with alert_id, detected_at, camera_id, target, fault_type, confidence, frame_id, model_version, and evidence path. | Must |
| FR-ALT-3 | The pipeline shall expose a local REST API for reading alert data. | Must |
| FR-ALT-4 | The pipeline shall expose a local web viewer on the server through the REST API. | Must |
| FR-ALT-5 | The viewer shall show alert time, fault, confidence, camera, and evidence image. | Must |
| FR-ALT-6 | The MVP shall not send push notifications or call an external notification backend. | Must |

## 3.5 Evidence and storage

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-EVD-1 | For every alert, the pipeline shall save one full-frame JPEG with all detections for that frame overlaid. | Must |
| FR-EVD-2 | The overlay shall use full-frame coordinates, draw boxes in red, and label each box with fault and confidence. | Must |
| FR-EVD-3 | Evidence shall be stored on the local server disk, not NAS/NFS, using a date and camera directory layout. | Must |
| FR-EVD-4 | The JPEG quality shall start at 85 and be configurable. | Must |
| FR-EVD-5 | The MVP shall retain evidence and alert rows for 365 days, subject to available disk space. | Should |

## 3.6 Resilience and shutdown

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-RES-1 | The system shall continue sampling while transient individual frames fail. | Must |
| FR-RES-2 | The system shall stop completely when a required dependency remains unavailable for the configured failure period. The initial value is five minutes. | Must |
| FR-RES-3 | The system shall log the reason and timestamp when it stops because of dependency failure. | Must |
| FR-RES-4 | The system shall restart using Docker Compose after an operator or maintainer starts it again. | Must |
| FR-RES-5 | The MVP shall not persist unprocessed frame queues across restarts. | Must |

# 4 External interfaces

| Interface | MVP contract |
| --- | --- |
| NVR | Dahua NVR channel 1 over Ethernet/RTSP. Exact IP, URL, credentials, codec, and firmware are deployment values. |
| Model | gRPC streaming on the local Docker network, no TLS in MVP. Full-frame JPEG request; structured detection response. |
| REST API | Local HTTP API bound to 127.0.0.1. No authentication in MVP. |
| Web viewer | Local HTTP page opened on the Ubuntu server at the configured localhost port. |
| Storage | Local ext4 filesystem on the server's 500 GB HDD. |

# 5 Non-functional requirements

| Category | MVP target |
| --- | --- |
| Latency | A qualifying detection should appear in the local viewer within a few seconds of model response. |
| False positives | Prefer low false-positive behavior; begin with confidence threshold 0.85 and tune using plant footage. |
| Availability | Run unattended during the test/operating window; automatic recovery and production SLA are deferred. |
| Scalability | Configuration and package structure shall support enabling CAM-2 through CAM-6 later without redesign. |
| Security | No network exposure beyond the server in MVP; no TLS/auth accepted only because the viewer is local. |
| Observability | Logs shall include camera_id, frame_id, model status, inference latency, queue drops, and stop reason. |
| Maintainability | Pipeline and model remain separate containers with independently versioned images. |

# 6 Alert data model

| Field | MVP rule |
| --- | --- |
| alert_id | UUID v4; unique per qualifying detection. |
| detected_at | Plant-local ISO 8601 timestamp with offset. |
| camera_id | CAM-1 for MVP. |
| observation_target | godet. |
| fault_type | CRACK or MISALIGNMENT for the intended MVP model output. |
| confidence | Raw model value 0–1; pipeline threshold is configurable. |
| bbox | Normalized x, y, width, height relative to the full frame. |
| frame_id/model_version | Traceability to the exact model request and build. |
| evidence_ref | Local relative path to the saved full-frame JPEG. |
| seen_at | One shared timestamp; optional local viewer behavior retained for later implementation. |

# 7 Detection scope and future phases

| Phase | Camera/target | Scope |
| --- | --- | --- |
| MVP | CAM-1 / godet | Misalignment and crack detection. |
| Phase 2 | CAM-2 / godet | Additional godet coverage after MVP validation. |
| Phase 2 | CAM-3 and CAM-4 / galet | Galet damage and missing/count checks. |
| Phase 2 | CAM-5 / clinker level | Level threshold and spillage. |
| Future | CAM-6 | Reserve or expansion camera. |

# 8 Open items before field deployment

| ID | Item | Owner |
| --- | --- | --- |
| OI-1 | Verify exact NVR model, firmware, IP address, web login, CAM-1 RTSP URL, and codec. | Project owner / plant |
| OI-2 | Record exact server CPU model, hostname, static IP, and network adapter layout. | Project owner |
| OI-3 | Obtain the model Docker image, proto, test images, and CPU benchmark. | Model owner |
| OI-4 | Confirm the local web port and exact REST response shape. | Project owner / developer |
| OI-5 | Measure real inference latency and decide whether the two-second sample interval is safe. | Project owner / model owner |
| OI-6 | Confirm local disk retention and cleanup behavior after observing actual alert rate. | Project owner |
| OI-7 | Define the exact five-minute dependency-stop behavior and operator restart procedure. | Project owner |

# 9 Out of scope for MVP

VMware VM deployment.

NAS/NFS storage and remote evidence serving.

Mobile application, push notifications, and external backend.

TLS, user authentication, and remote web access.

Processing all six cameras simultaneously.

Persistent frame replay after model failure.

SCADA/PLC control or automatic equipment shutdown.

Model training, model architecture, and final accuracy certification.